PDMS Substrate Ablation

Miller Lab

Jacob Albritton

Equipment/Materials

Procedure

- Turn on the laser water pump. (Very important!)

- Verify that the laser water pump is running! The laser can overheat without cooling.

- Verify that you verified that the laser water pump is on. (3 times)

- Turn on water pump, then the laser control unit (not the laser). To switch an emergency stop button on, twist the button clockwise about a quarter turn until the whole button releases outward. The RAMBo board will turn on with the laser control unit.

- Plug the RAMBo USB cable to a Pronterface enabled computer.

- Start pronterface. After opening the terminal, navigate to the folder containing pronterface.py using the “cd” command. Use the “python” command with “pronterface.py” to open pronterface. Currently on the AMRI-Two lab computer, after opening the terminal, the following command should open pronterface:

  - python sls/Printrun/pronterface.py

- On pronterface, click on Connect to get the RAMBo board online. Make sure the Baudrate is set at 115200 (top dropdown box next to “@” symbol). You may also have to check different Com ports to get the right USB port.

- Test the connection by clicking on the home all button on Pronterface (bottom left button with house symbol). The laser head should move to the endstops in response.

- Click on Load, and navigate to the .gcode file you have generated.

- Manually move the laser head to the desired starting position using pronterface arrows labeled “+x”, “-x”, “+y”, and “-y”.

- Turn on the laser.

- On pronterface, click print and wait until the laser starts ablating your sample.

- Once the print is done turn off the machine in the following order

  - Laser Power

  - Laser Cutter Control

  - Air Pump/Water Pump

- Turn water pump and unplug the air pump.